Performance Degradation on Large Apertures

Due to Ice Buildup

By Dick Knadel K2RIW & Mike Feher

Can anyone recommend a reference that would address the subject for
frequencies between 7 and 30 GHz? I realize that it would have to be an
approximation as there are too many variables, but, a fairly thorough Google
search on my part did not yield any results. I am familiar with performance
degradation due to tolerance variations across the surface as a function of
wavelength, and, expect ice to be somewhat similar, but, not certain.

Thanks  - Mike B. Feher, N4FS

To N4FS & The Microwavers de K2RIW  3/03/10

Mike,

Water has a dielectric constant of about 81 at most RF frequencies.  It is a 
very RF-absorptive substance.  That's primarily how your Microwave Oven (uw) 
works.  With a proper layout, water makes a good uw dummy load.

But, when water freezes, it becomes an entirely different substance that has 
almost no RF absorption characteristics, and a Dielectric Constant of about 
3.  Place an ice cube, or a snowball in your uw oven, alongside a glass of 
water, and you'll discover that only the glass of water experiences any RF 
heating.  That gives you some idea why the uw oven has a so-called Frozen 
Food Defrost cycle mode of operation.

The exact RF properties of water, ice, and many other substances are contained 
in an old, large format book authored by Von Hipple (I'm not sure of the 
spelling).

For a rough approximation, I think you assume that your blob of ice has a 
Dielectric Constant of 3, and a very low Loss Tangent.  You could literally 
make a double convex lens out of ice, and place it in front of your feed 
horn, if you desired to do RF focusing.

There are many parabolic dish antenna Ground Stations that seem to perform 
very well with a considerable amount of ice and snow on their surface.  I 
believe the major problem would be an ice build up on the feed horn causing a 
lensing effect.

73 es Good SHF/EHF DX,
  Dick, K2RIW

Dick, -

Thank you for a very common sense approach to give me a better handle on
this issue. It sounds like snow buildup would be a lot worse, and, I know
that from experience from my home brew TVRO receiver at 4 GHz with an 8 foot
Andrew dish back in the late 70's. The question has come up as a consequence
of whether or not to place de-icers on some 10 meter military satellite
dishes, which are in a relatively moderate climate operating from X through
Ka bands. Sounds like it would be a waste of taxpayer's money. I hate to
build things to cover the 3 sigma type situations. Thanks again & 73 - Mike

Mike  - looks like you got a good answer.  i looked thru the  antenna 
design articles i have copied over the years, but nothing.  i may  have made some 
observations years ago of the effect of snow on a radome.   We had 'snow 
ropes' to solve that problem.  the snow came falling down on  top of you when 
using them.

Will W0EOM